# Title Slide

Good afternoon I’m Alan Hinds and I’ll be presenting how MPLS is seen by Optus Systems Pty. Ltd.

# Agenda Slide

- Start by looking at how CWO sees MPLS

- Move to how MPLS has been deployed so far

- Finish by looking at our plans for using MPLS in the future

# Slide CWO Perspective on MPLS

- Technologies used in the carrier environment for new services or increasing efficiency or both

- Currently no plans to provide native MPLS services

- MPLS  seen as a way increasing efficiency and thereby reducing costs

- MPLS can carry multiple protocols, this can reduce the number of networks we have to build and support

- MPLS incorporates layer 2 and layer 3 functions.  This allows us to reduce the complexity of the network and potentially reduce the number of devices in the network.

# Slide MPLS adoption

- In this part of the presentation I’ll go thru where CWO is deploying MPLS

# Slide Deployment Strategy

- The deployment strategy is to gradually introduce the technology into the network.

- This is driven by availability of technology and the ability of the organisation to absorb change.

# Slide CWO MPLS Implementation Internet

- CWO has deployed an MPLS network

- This network is currently carrying core internet traffic

- Using MPLS paths the segregate national and international traffic – billed at separate rates hence need separate networks

- Used to use ATM PVC for this but MPLS is more efficient as we don’t need to hard provision traffic to each part of the network.  Also easier to provision

- The network use gigabit routers and a mixture of POS and GE transport

- POS is used between capital cities and GE over DWDM is used within cities

- Taking advantage of the MPLS fast re-route capability

- Allows us to provide same level of protection on both GE and POS

- Allows us to use both halves of an SDH ring under normal conditions

- Been running for almost 6 months without incident

Slide MPLS Implementation IP-VPN

Optus has some of its IP VPN running over MPLS

- Using RFC2547 approach with MP-BGP and label stacks

- Currently using MPLS over ATM in frame mode

- Migrating to the GE and POS based MPLS core being used by the Internet network

- ATM was used initially as this network was built before the MPLS core came into existence

- Currently we are only providing direct VPN over direct connects using the MPLS VPN

- Other VPN services such as IPSec tunnel termination and NAT are still being provided using the older techniques

# Slide Network Diagram

- This slide shows how the network will look in August when we finish our current build

- We currently have MPLS core nodes in sydney melbourne and adelaide and we are in the process of deploying additional nodes in brisbane and sydney

- Internet POPs and MPLS-VPN POPs are mostly co-located with the MPLS nodes however some are long lined to the nearest MPLS core node

# Challenges Slide

- There have been a number of challenges in implementing the MPLS network

- Most of these have been in the MPLS-VPN area, the internet part was relatively straight forward

- Provisioning systems for MPLS-VPN are immature.

- There is no auto discovery.  You  can’t extract pre-existing VPN from an existing network and have the system acquire this data

- The provisioning systems don’t support some functionality well e.g. QoS provisioning

- Performance reporting is limited

- The GUI assume a certain workflow which may not fit the organisation

- The other challenge has been that some functionality that exists in standard routers is not supported directly onto MPLS-VPN in particular NAT, IPSec

- This means that we need to install additional devices to implement these functions and have to connect via VLANs or PVCs into the MPLS-VPN

- Hopefully these issues will be resolved as the technology matures

# Future Developments

- Looking forward beyond the immediate deployment of MPLS there are a number of possibilites for further use of MPLS under consideration

- CWO ICON

- The technical direction for optus networks is the Integrated Converged Optus Network or ICON for short

- The goal under icon is to reduce from the existing multiple voice and data networks down to a single data network

- MPLS is expected to be the underlying “convergence” technology

- The following set of slides go through some of the potential developments that could lead to the ICON goal

# Short Term - VPN

- In the short term we plan to start using MPLS-VPN technology to allow us to stop dedicating routers to particular services

- Currently we have edge routers dedicated to National internet, international internet, bundled internet and VPN

- U ing MPLS-VPN we hope to be able to configure any one of these services on any edge router by assigning the port to the VPN associated with the service

- This will be a major step forward as it will reduce the number of devices in some areas and make planning and growing  the network considerably easier.

- There will be challenges with controlling the size of the route table however we think this can be overcome

# Short Term – VLAN

- Another development we are looking at in the short term is VLAN trunking over MPLS

- The main application is connecting VLANs across the Metro area

- This can be applied for both our extending our own internal network LANs and for implementing services such as transparent LANs and etherent interfaces for delivering Interent and IP services

- The major advantage of VLAN over MPLS is that it allows us to share transmission and edge devices with IP-VPN and Internet services

# Medium Term – ATM

- Looking a bit further out we are investigating the use of ATM trunking over MPLS

- Applying this technology would allow our ATM and IP networks to share the same core infrastructure

- This should bring reductions in the overall cost and complexity of the network

- We’re actively investigating the various implementations that have been built or are under development

- We see a number of significant challenges in moving to this, in particular

- QoS – can a network of MPLS routers support the QoS requirements of ATM and the Class or Service requirements of IP.

- Working out how much bandwidth will be required to meet the QoS and CoS

- Management – how can we achieve end to end point and click provisioning and management of and ATM network and the MPLS router  network without major system integration work

# Long Term – Voice

- In the longer term we are also looking at moving our voice networks onto a packetised network.

- Currently voice over  IP over MPLS  looks like the most promising approach however directly over MPLS may be possible

- Again there are significant challenges not the least of which is getting 5 or 6 nines availability out of and IP

- If we can get this all going then we will be very close to achieving optus icon goal of having a single network carrying all services

# Summary

- In summary we see MPLS as being a key technology

- It’s in service carrying Interent and some IP-VPN services

- We plan to use MPLS for VLAN and ATM trunking

- We are considering the use of MPLS for voice